🔹 Line 0 (Satellite Name)

| Parameter | Description | Example |
| --- | --- | --- |
| Satellite Name | Name/identifier of satellite | STARLINK-1008 |

🔹 Line 1 Parameters

| Parameter | Description | Example |
| --- | --- | --- |
| Line Number | Identifies line (always 1) | 1 |
| NORAD ID | Unique satellite ID | 44714 |
| Classification | Security classification | U |
| International Designator | Launch year & piece | 19074B |
| Epoch Time | Time of orbital data (YYDDD format) | 26097.14472000 |
| First Derivative of Mean Motion | Rate of change of speed | .00093530 |
| Second Derivative of Mean Motion | Acceleration of speed | 00000+0 |
| BSTAR Drag Term | Atmospheric drag factor | 27708-2 |
| Ephemeris Type | Orbit model type | 0 |
| Element Set Number | Version of data | 999 |
| Checksum | Error-check value | 8 |

🔹 Line 2 Parameters (MOST IMPORTANT)

| Parameter | Description | Example |
| --- | --- | --- |
| Line Number | Identifies line (always 2) | 2 |
| NORAD ID | Satellite ID (same as Line 1) | 44714 |
| Inclination (°) | Tilt of orbit | 53.1559 |
| RAAN (°) | Orbit orientation | 46.1506 |
| Eccentricity | Shape of orbit (0 = circular) | 0.0003232 |
| Argument of Perigee (°) | Orbit rotation | 126.1394 |
| Mean Anomaly (°) | Satellite position in orbit | 233.9908 |
| Mean Motion (rev/day) | Speed of orbit | 15.34101796 |
| Revolution Number | Total orbits completed | 35323 |